# Verification Test Report

Thermodynamic Analysis of a Gas in a Closed System with a Movable Piston

Author: [Your Name/Department]

Date: October 26, 2023

Document Code: TH-AN-001

Executive Summary

This report provides a comprehensive thermodynamic analysis of a classical closed system: an ideal gas confined within a cylinder equipped with a frictionless, movable piston. The system serves as a foundational model for understanding energy conversion principles in engineering. The analysis is anchored in the First Law of Thermodynamics and the Ideal Gas Law. Key findings detail the mathematical formulation for boundary work (W = ∫ P dV) and its path-dependent nature, which is rigorously illustrated through the examination of isobaric, isochoric, isothermal, and adiabatic processes. The report establishes that for an ideal gas, internal energy is a function of temperature alone, a critical simplification. Furthermore, it synthesizes how the piston acts as the essential mechanism for converting heat transfer into useful mechanical work and vice versa. The inclusion of polytropic processes and real-gas considerations extends the model's applicability. This analysis is directly relevant to the design and evaluation of internal combustion engines, compressors, and other fundamental thermodynamic devices.

1. Introduction and System Definition

The piston-cylinder assembly is the most ubiquitous conceptual device in classical thermodynamics, providing a tractable model for analyzing energy interactions in a closed system (fixed mass). The primary objective of this report is to perform a rigorous thermodynamic analysis of a gas within such a device, deriving the governing equations for work and heat transfer, and demonstrating their application under standard process conditions.

The system is defined as the gas enclosed by the cylinder walls and the piston crown. It is a closed system, meaning no mass crosses its boundary. The piston is assumed to be massless, frictionless, and perfectly sealed, allowing only for energy interactions via work and heat.

Key Interactions:

Work (W): Energy transfer due to the displacement of the piston under a pressure difference. This is boundary work.

Heat (Q): Energy transfer due to a temperature difference across the cylinder walls.

Mass Transfer: Zero by definition of a closed system.

The state of the system is defined by properties: Pressure (P), Volume (V), and Temperature (T). A process is a change from one equilibrium state to another.

2. Foundational Laws and Concepts

2.1 Zeroth Law: Establishes the concept of temperature and thermal equilibrium, allowing the use of a thermometer to define the system's temperature.

2.2 First Law of Thermodynamics (Energy Conservation):

For the closed system, the First Law is expressed as:

ΔU = Q - W ... (1)

Where:

ΔU = Change in internal energy of the system [J]

Q = Net heat transfer into the system [J]. (Sign Convention: +Q for heat addition, -Q for heat rejection).

W = Net work done by the system [J]. (Sign Convention: +W for work done by the system on the surroundings, -W for work done on the system).

This law is the cornerstone of the analysis, stating that the net energy transfer is stored as internal energy. The sign convention adopted is the engineering standard.

3. The Work Interaction: Boundary (P-V) Work

The work required to move the piston is calculated by considering an infinitesimal quasi-static expansion or compression.

3.1 General Formulation:

For a differential displacement, the boundary work is δW = F * ds = (P * A) * ds = P * (A ds) = P dV.

Therefore, the total work for a finite process from state 1 to state 2 is:

W = ∫₁² δW = ∫₁² P dV ... (2)

3.2 Critical Assumption - Quasi-Equilibrium Process:

Equation (2) is valid only for a quasi-static process, where the system remains infinitesimally close to an equilibrium state throughout. This ensures that the pressure P in the integral is the uniform system pressure. Non-quasi-static processes cannot be described by a simple P-V path and require more complex analysis.

3.3 Graphical Interpretation:

On a Pressure-Volume (P-V) diagram, the work W for a quasi-static process is represented by the area under the process curve between the initial and final volumes. This highlights the path-dependence of work—different processes between the same two states will result in different work values.

4. Analysis of Standard Thermodynamic Processes

For each process, we assume the working fluid is an ideal gas with constant specific heats, unless otherwise stated. The relevant property relations are defined in Section 5.

4.1 Isobaric Process (Constant Pressure)

Governing Constraint: P = constant

Work from Eq. (2): W = P ∫₁² dV = P (V₂ - V₁) ... (3)

First Law Application: ΔU = Q - PΔV → Q = ΔU + PΔV = ΔH (from enthalpy definition).

P-V Diagram: A horizontal line. Work is the rectangular area under the line.

4.2 Isochoric Process (Constant Volume)

Governing Constraint: V = constant, dV = 0

Work from Eq. (2): W = 0 ... (4)

First Law Application: ΔU = Q (All heat transfer changes the internal energy).

P-V Diagram: A vertical line. Area under the line is zero, signifying no work.

4.3 Isothermal Process (Constant Temperature)

Governing Constraint: T = constant. For an ideal gas, ΔU = 0 (as shown in Sec. 5.2).

Work Calculation: Using P = nRT/V in Eq. (2):

W = ∫₁² (nRT/V) dV = nRT ∫₁² (dV/V) = nRT ln(V₂/V₁) ... (5)

Since P₁V₁ = P₂V₂ = nRT, alternative forms are: W = P₁V₁ ln(V₂/V₁) = P₁V₁ ln(P₁/P₂).

First Law Application: ΔU = 0 = Q - W → Q = W. All heat added is converted to work done by the system.

P-V Diagram: A hyperbolic curve (P ∝ 1/V). Work is the area under the hyperbola.

4.4 Adiabatic Process (No Heat Transfer)

Governing Constraint: Q = 0

First Law Application: ΔU = -W. Work is done at the expense of internal energy.

Relation between Properties: For an ideal gas with constant specific heats, derivation using δQ = dU + PdV = 0 yields:

P V^γ = constant ... (6a)

T V^(γ-1) = constant ... (6b)

T P^((1-γ)/γ) = constant ... (6c)

where γ = c_p / c_v is the adiabatic index (≈1.4 for air).

Work Calculation: Using Eq. (6a) (P = C / V^γ) in Eq. (2):

W = ∫₁² (C / V^γ) dV = C (V₂^(1-γ) - V₁^(1-γ)) / (1-γ)

Substituting C = P₁V₁^γ = P₂V₂^γ simplifies to:

W = (P₂V₂ - P₁V₁) / (1 - γ) = (P₁V₁ - P₂V₂) / (γ - 1) ... (7)

Using the Ideal Gas Law, this can also be expressed as:

W = m R (T₁ - T₂) / (γ - 1) = m c_v (T₁ - T₂) (consistent with ΔU = -W).

P-V Diagram: A steeper curve than the isotherm (P ∝ V^(-γ)). The area under this curve for a given ΔV is less than for an isothermal process.

5. Modeling the Gas: Ideal Gas Law and Property Relations

5.1 Equation of State:

The ideal gas model is employed: P V = n R_u T or P V = m R_s T ... (8)

P: Absolute pressure [Pa]

V: Volume [m³]

n: Number of moles [mol]

R_u: Universal gas constant (8.314 J/mol·K)

T: Absolute temperature [K]

m: Mass [kg]

R_s: Specific gas constant (R_u / M, where M is molar mass) (e.g., 287 J/kg·K for air)

5.2 Internal Energy and Enthalpy:

Joule's Law: For an ideal gas, internal energy depends solely on temperature: U = U(T).

Constant Specific Heats: ΔU = m c_v ΔT and ΔH = m c_p ΔT ... (9)

where c_v and c_p are specific heats at constant volume and pressure, respectively, and are related by: c_p - c_v = R_s and γ = c_p / c_v.

Enthalpy: Defined as H = U + PV. For an ideal gas, H = U + (m R_s T), therefore H = H(T) also.

6. Core Arguments and Synthesis

Path Dependence Demonstrated: A gas expanding from (P₁, V₁) to (P₂, V₂) will perform maximum work if the process is isothermal (heat is supplied to maintain T), less work if adiabatic (temperature drops), and a different amount if taken via an isobaric-isochoric combination. This irreducibly proves that work and heat are process-dependent functions, not state properties.

Energy Conversion Mechanism: The movable piston is the enabling component that translates the random molecular motion of a heated gas (internal energy) into organized macroscopic motion (boundary work). The First Law quantitatively balances this conversion.

Centrality of the P-V Diagram: This diagram is the principal engineering tool for visualizing processes, comparing work magnitudes (as areas), and analyzing thermodynamic cycles. The sequence of processes forms a closed loop, the net area of which represents net cycle work.

7. Advanced Considerations & Limitations

7.1 Polytropic Process (Generalization):

Many real processes are described by P V^n = constant, where n is the polytropic exponent.

n = 0 → Isobaric

n = 1 → Isothermal (for ideal gas)

n = γ → Adiabatic (for ideal gas with constant c_p, c_v)

n = ∞ → Isochoric

The work formula is analogous to the adiabatic work (Eq. 7), with γ replaced by n:

W = (P₁V₁ - P₂V₂) / (n - 1) ... (10)

7.2 Real Gas Behavior:

At high pressures and/or low temperatures, intermolecular forces become significant. The Ideal Gas Law is replaced by complex equations of state (e.g., Van der Waals: [P + a/V_m²][V_m - b] = R_u T). The compressibility factor Z = PV/(nR_uT) is introduced, where Z ≠ 1 indicates deviation from ideal behavior.

7.3 Irreversibilities:

The assumption of a frictionless piston and quasi-static operation is ideal. In reality, friction causes some work to be dissipated as heat, and finite pressure gradients create non-equilibrium states. For such irreversible processes, the work done by the gas W_actual is less than the calculated ∫ P dV using the system's equilibrium pressure, i.e., W_actual < ∫ P_sys dV.

8. Practical Applications & Conclusion

8.1 Applications:

Internal Combustion Engines: The power stroke is modeled as an adiabatic expansion (or polytropic with n ~ 1.3).

Reciprocating Compressors: The compression stroke is analyzed as isentropic (ideal adiabatic) or polytropic.

Steam Engines: Piston-cylinder work from expanding steam, though phase changes must be considered.

Closed Thermodynamic Cycles: The foundational component for analyzing Otto, Diesel, Brayton, and Stirling cycles.

8.2 Conclusion

The analysis of a gas within a piston-cylinder device successfully encapsulates the fundamental principles of classical thermodynamics. The derived mathematical formulations—centered on the First Law and the boundary work integral—provide a complete framework for calculating energy interactions. The critical understanding that work and heat are path-dependent, visualized through the P-V diagram, is a key outcome. While the ideal gas model and quasi-static assumptions provide a powerful analytical foundation, the extensions to polytropic processes and the acknowledgment of irreversibilities and real gas effects are necessary for applying this analysis to real-world engineering systems. This model remains the indispensable starting point for all thermodynamic studies of closed systems.

References:

Cengel, Y. A., & Boles, M. A. Thermodynamics: An Engineering Approach. McGraw-Hill.

Moran, M. J., Shapiro, H. N., Boettner, D. D., & Bailey, M. B. Fundamentals of Engineering Thermodynamics. Wiley.

Sonntag, R. E., Borgnakke, C., & Van Wylen, G. J. Fundamentals of Thermodynamics. Wiley.